RV32I 5-Stage Pipelined Processor

RTL Issue Audit and Correction Plan

Student / Research Prototype Review

Scope: fundamental functional correctness, pipeline hazards, memory interface behavior, and basic SoC integration

Audit basis: supplied RTL + current directed full-processor test + simple RV32I architectural reference model

18 September 2026

# 1. Executive Summary

Bottom line: Three RTL issues are presently strong candidates for direct correction: (1) the branch signed/unsigned compare mapping is reversed in the ALU decoder, (2) subword-store data is not aligned consistently with the byte write strobe, and (3) the EX forwarding path forwards ALUResultM even for instructions whose architectural write-back value is PC+4 (JAL/JALR).

# 2. Severity Definitions

| Severity | Meaning for this project |
| --- | --- |
| MAJOR | A functional or integration defect that can make a supported instruction class produce architecturally wrong results, corrupt state, or prevent reliable full-processor operation. Fix before moving to broad verification. |
| MINOR | A real design weakness that normally does not break the current intended instruction set, but causes avoidable stalls, fragile behavior, missing error reporting, or unnecessary integration risk. |
| NEGLECTABLE | Cleanup, style, documentation, or low-impact structural issue that does not presently threaten the research prototype's basic functional goal. Fix only after major/minor correctness work. |

# 3. Simple Architectural Reference Model

| Instruction class | Reference result / rule |
| --- | --- |
| R-type arithmetic | ADD/SUB operate on 32-bit values; SLL uses rs2[4:0]; SRL is logical; SRA is arithmetic. |
| SLT / SLTI | Signed comparison. |
| SLTU / SLTIU | Unsigned comparison. |
| SB | Store rs2[7:0] to memory byte at address. |
| SH | Store rs2[15:0] to the two addressed bytes, little-endian. |
| SW | Store all 32 bits of rs2 at the aligned word location. |
| LB / LBU | Read the addressed byte and sign-extend / zero-extend. |
| LH / LHU | Read the addressed halfword and sign-extend / zero-extend. |
| LW | Read the complete 32-bit word. |
| BEQ / BNE | Equality / inequality of register values. |
| BLT / BGE | Signed less-than / signed greater-than-or-equal. |
| BLTU / BGEU | Unsigned less-than / unsigned greater-than-or-equal. |
| JAL | rd = PC + 4; target = PC + J-immediate. |
| JALR | rd = PC + 4; target = (rs1 + I-immediate) with bit 0 cleared. |
| LUI | rd = U-immediate. |
| AUIPC | rd = PC + U-immediate. |

Scope note: The supplied expected-results file identifies 37 unique supported RV32I instructions. The current directed test prints 41 checks because some memory and jump behaviors are checked separately.

# 4. MAJOR RTL ISSUES

These are the current issues that should be treated as actual RTL correction work.

RTL-M1 - Signed and unsigned branch compare controls are reversed

| SEVERITY | MAJOR |
| --- | --- |
| Block / Stage | ALU Decoder / EX Branch decision |
| Exact source | `ALU_Decoder(7).sv`, lines 21-24 and lines 40-41. Branches map BLT/BGE to ALUControl=1001 and BLTU/BGEU to 1000, while the R-type mapping shows 1000=SLT (signed) and 1001=SLTU (unsigned). `Execute_stage(7).sv`, lines 198-201 then uses `less_thanE` for BLT/BGE and its inverse for BGEU. |
| Observed symptom | The full-processor test gives BLT=0, BGE=0, and BGEU=0 where the selected operands require these branches to be taken. The standalone ALU verification had already passed both signed and unsigned comparison behavior, so the error is in decoding, not in the ALU itself. |
| Why this is an issue | The decoder selects the wrong comparison operation for all four signed/unsigned branch variants. For example, BLT must use the signed compare control and BLTU must use the unsigned compare control. |
| Suggested correction | Change the branch mapping to: funct3 100/101 -> 4'b1000 (signed compare); funct3 110/111 -> 4'b1001 (unsigned compare). Keep the EX branch logic unchanged. |
| How to verify | Run isolated branch tests first, with operands chosen so signed and unsigned results differ. Then rerun the full directed program. BLT/BGE/BLTU/BGEU should all be checked independently in taken and not-taken cases. |

RTL-M2 - Subword store WDATA is not aligned consistently with WSTRB

| SEVERITY | MAJOR |
| --- | --- |
| Block / Stage | DMEM Stage + AXI RAM Slave / byte-lane interface |
| Exact source | `data_memory(5).sv`: line 33 drives `MEM_WDATA = WD`; lines 60-62 shift WSTRB by A[1:0]. `axi_ram_slave(1).sv`: lines 202-209 copy the same WDATA byte lane selected by WSTRB into the RAM word. |
| Observed symptom | The directed test stores x29 = 0xFFFFFFF9 at address 0x106. WSTRB correctly selects byte lane 2, but WDATA remains 0xFFFFFFF9, so byte lane 2 receives WDATA[23:16] = 0xFF instead of the architectural store byte 0xF9. The observed word becomes 0x00FFFFF9 instead of 0x00F9FFF9. |
| Why this is an issue | For SB/SH, the architectural data quantity is the low 8/16 bits of rs2, but the physical AXI byte lanes are selected by WSTRB. The design currently assumes both sides will interpret the same unshifted WDATA correctly, which is false for nonzero byte offsets. |
| Suggested correction | Choose one clear interface convention and use it end-to-end. The simplest student-level fix is to align the store data toward the selected byte lanes before sending WDATA, or alternatively make the AXI slave place the low 8/16 bits into the lanes selected by WSTRB. Do the same for the testbench model. |
| How to verify | Test SB at offsets 0,1,2,3; SH at aligned offsets 0 and 2; and SW. After each store, perform a full LW and compare every byte of the word. Then test loads from each modified byte/halfword. |

RTL-M3 - M-stage forwarding forwards ALUResultM even when the architectural result is PC+4

| SEVERITY | MAJOR |
| --- | --- |
| Block / Stage | Hazard Unit + EX forwarding path |
| Exact source | `hazard_unit(10).sv`, lines 43-59 selects M-stage forwarding whenever RegWriteM and the destination register matches. `Execute_stage(7).sv`, lines 110-129 feeds `ALUResultM_forward` into the forwarding mux. `pipeline_top(2).sv`, line 182 connects `ALUResultM_forward` directly to `ALUResultM`. `main_decoder(8).sv`, lines 73-85 sets JAL/JALR ResultSrc=2'b10, meaning the write-back value is PC+4; `write_back_stage(6).sv` lines 15-21 confirms ResultSrc=10 selects PcPlus4W. |
| Observed symptom | JAL and JALR target-path checks pass, but link-value checks fail. A dependent instruction that reaches EX while JAL/JALR is in M receives ALUResultM rather than the value that will actually be written to the destination register. |
| Why this is an issue | The forwarding path must forward the producer's architectural result, not simply its ALU output. JAL/JALR do not define their destination result as the ALU result. |
| Suggested correction | Create an M-stage forward-data mux. For ResultSrcM=00 forward ALUResultM; for ResultSrcM=01, do not treat a load as immediately forwardable from M (the existing load-use stall can remain); for ResultSrcM=10 forward PcPlus4M. Feed that selected value to the M-source of the EX forwarding muxes. |
| How to verify | Create a minimal test with JAL rd followed by an instruction that uses rd as rs1/rs2, and the same for JALR. Also test normal ALU-result forwarding to confirm the existing path still works. |

# 5. MINOR RTL ISSUES

RTL-m1 - Load-use detection stalls some instructions unnecessarily

| SEVERITY | MINOR |
| --- | --- |
| Block / Stage | Hazard Unit |
| Exact source | `hazard_unit(10).sv`, lines 70-73 stalls whenever RdE matches Rs1D or Rs2D for any load in E, without checking whether the current D-stage instruction actually reads those fields. |
| Observed symptom | Some instructions can have encoded Rs1/Rs2 bit fields that are not architecturally used. They can therefore cause a needless one-cycle stall. |
| Why this is an issue | This is primarily a performance issue, not a current correctness failure. |
| Suggested correction | Keep the simple hazard rule for now if simplicity is the priority. Later, qualify the comparison with instruction-use signals generated by Decode. |
| How to verify | Add a performance counter or simple waveform check to confirm the extra stalls are harmless before optimizing them. |

RTL-m2 - AXI Manager ignores AXI response error codes

| SEVERITY | MINOR |
| --- | --- |
| Block / Stage | AXI Manager |
| Exact source | `axi_manager(1).sv` accepts `axi_bresp` and `axi_rresp` as inputs, but the write/read completion states only wait for valid/ready and then assert `mem_ready` (lines 159-187). |
| Observed symptom | An invalid address can produce SLVERR in the slave, but the CPU-facing interface has no error bit, so the transaction can still look successful to the processor. |
| Why this is an issue | For the present small research prototype and valid-address directed tests, this does not explain the current failures. It does mean basic bus errors are silently converted into successful CPU transactions. |
| Suggested correction | Keep the current interface if desired, but document that only successful, in-range addresses are supported. A later improvement can add a one-bit memory error output and block write-back on failed loads. |
| How to verify | Run one invalid-address test and observe that the behavior is documented rather than silently treated as a normal access. |

RTL-m3 - Instruction memory reset-valid gating uses a blocking assignment inside always_ff

| SEVERITY | MINOR |
| --- | --- |
| Block / Stage | IMEM interface / coding robustness |
| Exact source | `inst_memory(20260918-104605).sv`, line 24 uses `allow_instr = 1'b1` inside `always_ff` rather than nonblocking assignment. |
| Observed symptom | The current simulation can still work, but sequential state is conventionally updated with nonblocking assignments. Mixing styles makes the timing intent less clear and can complicate debugging. |
| Why this is an issue | This is not a demonstrated functional failure by itself. |
| Suggested correction | Change to `allow_instr <= 1'b1` and then re-run the IF test. Keep the timing contract explicitly documented. |
| How to verify | Run the dedicated IF test and check that the first valid instruction appears at the intended cycle. |

RTL-m4 - Commented pipeline-register priority does not exactly match implemented condition order

| SEVERITY | MINOR |
| --- | --- |
| Block / Stage | EX/MEM pipeline register |
| Exact source | `Execute_stage(7).sv`, comments at lines 223-239 claim StallM has higher priority than FlushE, but the `always_ff` at lines 260-277 checks `FlushE` before `StallE`. The current hazard unit masks FlushE with `~bus_stall`, so the mismatch is partly compensated by upstream control. |
| Observed symptom | A future change to hazard logic could make the documented and implemented priorities diverge and cause difficult-to-debug branch/memory interactions. |
| Why this is an issue | At present this is mostly a consistency problem rather than a proven failure. |
| Suggested correction | Either reorder the conditions to match the comment or rewrite the comment to match the intended control contract. Do not change behavior blindly; test a branch concurrent with a memory stall before finalizing. |
| How to verify | Add one directed test where a taken control transfer coincides with a multi-cycle data transaction. |

# 6. NEGLECTABLE / CLEANUP ISSUES

| Block | Issue | Why it is low priority |
| --- | --- | --- |
| EX Stage | Self-assignments such as `RegWriteM <= RegWriteM` during stall. | Legal but unnecessary. An omitted assignment in a clocked block already holds the register. |
| Control Unit | Unsupported opcodes default to no-op-like control. | Acceptable for the current reduced RV32I research scope. Exceptions/illegal-instruction handling are outside the present goal. |
| SoC / IMEM | Core and loader use byte-style PC/addressing while the 1024-word macro uses only 10 address bits. | Functional for the short current program; it mainly limits effective instruction-memory range and should be cleaned up later. |
| Documentation | Some comments still mention APB or old control assumptions not present in the visible AXI-only subsystem. | Documentation consistency problem, not a functional blocker. |
| Data_RAM model | Simple behavioral macro model and compact array declaration. | Adequate for a student simulation model; no current directed-test failure points to it. |

# 7. Stage / Block-Wise Audit Summary

| Block / Stage | Current status | Issue class | Action now |
| --- | --- | --- | --- |
| IF Stage | Timing path is designed around synchronous instruction read; current TB is combinational. | MAJOR verification issue; no confirmed core-RTL failure yet. | Fix TB memory timing first. |
| IMEM SRAM block | Registered SRAM behavior is consistent with synchronous read intent. `allow_instr` coding style is fragile. | MINOR cleanup. | Keep architecture; change blocking to nonblocking and retest. |
| Decode / Main Decoder | Supported opcode controls are internally consistent with current subset. | No confirmed issue. | Do not change now. |
| ALU Decoder | Branch funct3 mapping is reversed for signed/unsigned compares. | MAJOR RTL-M1. | Correct branch mapping and rerun branch tests. |
| EX Stage | ALU datapath works for many operations. JAL/JALR target generation works. M-stage forwarding source is incomplete for PC+4 results. | MAJOR RTL-M3. | Forward selected M-stage architectural result. |
| Hazard Unit | Basic forwarding and load-use stall structure is sensible. Some false stalls are possible. | MAJOR together with forwarding result bug; MINOR over-stalling. | Fix result-selection forwarding first; optimize stall qualification later. |
| DMEM Stage | Load extraction passes directed tests. Store strobe generation is straightforward. | MAJOR interface bug for SB/SH data placement. | Define and implement one byte-lane convention. |
| AXI Manager | Handshake FSM is adequate for a single outstanding simple transaction. | MINOR: response errors ignored. | Leave until basic data path is correct. |
| AXI RAM Slave | Basic read/write handshakes and partial-write RMW are present. | MAJOR as part of SB/SH lane contract; otherwise no current fundamental failure. | Fix WDATA/WSTRB interpretation consistently. |
| Write-back Stage | ALU/load/PC+4 selection is correct for current ResultSrc values. | No confirmed issue. | Do not modify. |
| Register File | x0 is protected; write-through read behavior is present. | No confirmed issue. | Do not modify. |
| PC Adder / PC Module / MUX / Sign Extend | Simple and consistent; no current failure points to these. | No confirmed issue. | Do not modify unless a targeted test fails. |
| SoC / Loader | Basic loader/reset/address wiring is understandable; address-width cleanup remains. | NEGLECTABLE/MINOR. | Postpone until processor core passes directed tests. |

# 8. What Should NOT Be Changed Right Now

Do not change the ALU simply because several arithmetic checks failed. The arithmetic pattern points to the instruction-memory timing problem, and the standalone ALU check already passed signed/unsigned comparison behavior and the shift cases used here.

Do not rewrite the Write-back stage. ResultSrc=10 correctly selects PC+4 for JAL/JALR.

Do not change the PC target logic only to make JAL/JALR link checks pass. The target checks pass; the link issue belongs to the forwarding result selection.

Do not change load formatting based on the current run. LW/LH/LHU/LB/LBU are passing, so the load path should be left stable while the store path is corrected.

Do not compensate for a wrong testbench expectation by changing RTL. In particular, BLTU should be checked against the architectural unsigned comparison, not against the current incorrect expected constant.

# 9. Recommended Correction Order

Fix the ALU Decoder branch mapping (RTL-M1). This is a small, direct RTL correction.

Fix the instruction-memory testbench to model the actual synchronous SRAM response (TB-M1). Then rerun the arithmetic, AUIPC, and PC-dependent checks before changing any more RTL.

Fix the expected-value infrastructure: one canonical HEX file and correct BLTU expectation (TB-M2/TB-M3).

Fix the SB/SH WDATA/WSTRB convention across DMEM, AXI slave, and TB (RTL-M2).

Fix M-stage forwarding so JAL/JALR forward PC+4 rather than ALUResultM (RTL-M3).

Only after the above passes, move to targeted hazard, multi-cycle memory, and control-transfer interaction tests.

# 10. Final Issue Register

| ID | Severity | Block | Issue | Status |
| --- | --- | --- | --- | --- |
| RTL-M1 | MAJOR | ALU Decoder | BLT/BGE vs BLTU/BGEU control mapping reversed. | Confirmed RTL bug. |
| RTL-M2 | MAJOR | DMEM + AXI | SB/SH WDATA lane alignment inconsistent with WSTRB. | Confirmed interface-level RTL bug from observed data. |
| RTL-M3 | MAJOR | Hazard + EX | M forwarding uses ALUResultM even when write-back source is PC+4. | Confirmed RTL design flaw; matches JAL/JALR link failures. |
| TB-M1 | MAJOR | IF/IMEM TB | Combinational TB model does not match synchronous SRAM timing used by DUT. | Confirmed verification mismatch; fix before further RTL diagnosis. |
| TB-M2 | MAJOR | Expected values | BLTU expected result is wrong for selected operands. | Confirmed checker error. |
| TB-M3 | MAJOR | Program control | Multiple HEX/expected files are inconsistent. | Confirmed infrastructure risk. |
| RTL-m1 | MINOR | Hazard Unit | Some unnecessary load-use stalls. | Design inefficiency; not a current correctness blocker. |
| RTL-m2 | MINOR | AXI Manager | AXI error responses are ignored. | Low priority for current valid-address research tests. |
| RTL-m3 | MINOR | IMEM | Blocking assignment inside always_ff for allow_instr. | Cleanup; not a demonstrated root cause. |
| RTL-m4 | MINOR | EX/MEM register | Commented priority does not match condition ordering exactly. | Documentation/control-contract cleanup. |
| N-01 | NEGLECTABLE | Various | Self-assignment on stalls, documentation drift, compact memory-model style. | Postpone. |